ICT projekti – Ilmanpaine

(signaali 32)

Tekninen dokumentaatio

Olli Laurila

Ari Savolainen

Valdemar Tuominen

SISÄLLYS

1	JOHDANTO	4

2	TEKNISEN DOKUMENTIN TAVOITTEET, RAJAUS JA TULOKSET	5

3	OHJELMIEN ARKKITEHTUURIN KUVAUS	6

3.1.	LCD-näytön testiohjelma	6

3.2.	Arduino ohjelma	6

3.3.	MQTT C++ ohjelma	6-7

3.4.	Raspberry ohjelma	7-9

4	FUNKTIOT	10-12

5	RAJAPINNAT	13

6	KAAVIOT, PIIRRUSTUKSET JA KUVAT	14-24

7	LIITTEET	25

JOHDANTO

Tämän dokumentaation tekemiseen johti ICT-kurssi, jonka opetuksesta osa toteutetaan projektiopetuksena johon tämä dokumentaatio liittyy.

DOKUMENTAATION TAVOITTEET, RAJAUS JA TULOKSET

Projektin tavoitteena on aikaan saada ilmanpaineanturin tietojen siirtyminen oikeassa muodossa TAMK:in serverille ja että ilmanpainetta pystytään lukemaan numeerisesti sekä graafisesti web-selaimella vaaditussa aikataulussa ja tekniset vaatimukset täyttäen.

Sekä oppia miten toimitaan työelämän projektityöympäristössä tehden yhteistyötä eri sidosryhmien kanssa, tiimin vetäjänä eli vastuuhenkilönä ja jäsenenä. Tämä tekninen dokumentaatio sisällytetään osaksi projektisuunnitelmaa.

Dokumentaatio on rajattu edellä mainittuihin projektin tavoitteisiin.

- 3. OHJELMIEN ARKKITEHTUURIN KUVAUS

| 3.1 | LCD-näytön testiohjelma |
| --- | --- |
| Kuvaus: | (Ks. koodikommentit liitteistä.) #include <LiquidCrystal.h> LiquidCrystal lcd(8, 7, 6, 5, 4, 3); void setup() { // put your setup code here, to run once: lcd.begin(16, 2); lcd.clear(); } void loop() { // put your main code here, to run repeatedly: lcd.setCursor(0,0); lcd.print("TEST"); |
| 3.2 | Arduino ohjelma |
| Kuvaus: paine_aika.ino Hae IP BMP.begin BMP.read_pressure printTime laheta_pyynto Connect_MQTT_server(); send_message(paine); | (Ks. koodi liitteistä.) - Arduino W5100 EtherCard ICT 2016 alkukoodit - edit 15.1.2016 KN - // Kytkeydytään TAMK:n verkkoon ja pyydetään IP-osoite. - Luodaan yhteys ilmanpainesensoriin. - // Adafruit kirjaston funktio joka palauttaa painedatan pascaleina. - // Kutsuu lähetä-pyyntö funktiota ja tulostaa sen LCD-näytölle muodossa hh:mm:ss - //Parsii ajanhakupyynnöstä saadun datan tulostettavaan aikamuotoon. - //Lähettää NTP-pyynnön palvelimelle josta palautuu EPOC-aika. - // Otetaan yhteys MQTT serveriin. - // Ottaa parametrina painelukeman ja lähettää sen MQTT serverille |

| 3.2 | MQTT C++ ohjelma |
| --- | --- |
| Kuvaus: | (Ks. myös koodikommentit liitteistä) |
| main.cpp | Vaateita 3kpl:tta. MQTT luku. LähetysTAMK:in serverille. Ajastin ajan lähettämiseksi. |
| mqtt_arduino.cpp | Arduinon lähettämän mittausdatan lukeminen ja MQTT protokollan käyttö. Keskustelun kuuntelu : outTopic Vastaanotetun datan tallentaminen globaaliin datapöytään table: data[40] Kaikkien signaalien mittausdata jaotellaan ja tallentuu A ja B luokan datana. |
| SendTamk.cpp | Kaiken signaalidatan lähettäminen serverille yhtä aikaa. Kaikki mittausdata lähetetään yhteen URL-osoitteeseen. Mittausdatan lähetys A-luokan ja B-luokan tietokantaan. CURL-kirjaston käyttö GET-pyynnön lähettämiseksi serverille. 1. Data kopioituu globaalista datapöydästä // data[] // paikalliseen datapöytään // apudata[] // 2. URL ylätunnisteen luonti // string url_id // 3. Jokainen datasignaali lisätään // url_id ) //:hen. 4. URL lähetetään // curl_easy_perform(hnd) // käskyllä. 5. // float ltoflo(int index) // Luetaan data paikallisesta datapöydästä // apudata[] // 6. Muutetaan pitkä integer data-arvo floatiksi, indeksinä on signaalinumero (=tiimi 32). 7. URL-lähetys A- ja B-luokka tietokantoihin. 8. Lähetysvaateet odottavat MQTT viestejä ennen URL:in lähettämistä serverille. |
| 3.3 | Raspberry ohjelma |
| Kuvaus: | (Ks. koodikommentoiti liitteistä.) |
|  | #define IP_tamk //#define IP_koti ////////////////// //// Raspberryn IP Koti #ifdef IP_koti #define IP_1 192 #define IP_2 168 #define IP_3 1 #define IP_4 7 #endif //// Raspberryn IP TAMK #ifdef IP_tamk #define IP_1 192 #define IP_2 168 #define IP_3 112 #define IP_4 23 #endif // apu_sek = asek % 120; // apu_sek 0-120, 0-120 .... // apu_sek = asek % osam; // apu_sek = apu_sek / 5; // 120/5 = 24 apu_sek 1-24 , 1-24, .... // apu_sek = apu_sek / jakaja; // apu_sek=asek % 15; // testausta varten nopeampi paivitys vali // // Testi moodi ///////////// //#define osam 15 // testi moodi 0-15, 0-15, o-15... sek //#define jakaja 1 // testi moodi // Mittaus //////////////// #define osam 120 // mittaus moodi 0-120, 0-120, o-120... sek #define jakaja 3 // mittaus moodi / 120/3 = 40 apu_sek 1-40 , 1-40, .... ///////////////////////////////////////////////////////////////////////////// #ifdef IP_koti char website[] = "time1.mikes.fi"; #endif #ifdef IP_tamk char website[] = "ntp.tamk.fi"; #endif //char website[]PROGMEM = "1.us.pool.ntp.org"; ///char website[]PROGMEM = "ntp2d.mcc.ac.uk"; //char website[]PROGMEM = "ntp2c.mcc.ac.uk"; //char website[]PROGMEM = "ntp.exnet.com"; //char website[]PROGMEM = "ntp.cis.strath.ac.uk"; //char website[]PROGMEM = "clock01.mnuk01.burstnet.eu"; |

- 4  FUNKTIOT

Arduino:     (Ks. koodikommentointi liitteistä)

1. // serial monitor on 2. // include LCD library 3. // incluide Ethernet library 4. // include MQTT library 5. // LCD pin wiring settings 6. // Ethernet configuration 7. // MAC nro 8. // Last mac number MSB mac numbers at ethernet_mega.c 9. // Not relevat with Ethershield 10. // MQTT settings 11. // MQTT port number 12. // * set your device id (will be the MQTT client username) 13. // * set a random string (max 23 chars, will be the MQTT client id)

14. // * set your device secret (will be the MQTT client password)

15. // MQTT Server settings 16. // HOME IP MQTT server address 17. // TAMK IP 18. // subscription callback for received MQTTT messages 19. // mqtt client 20. // MQTT topic names 21. // * MQTT channel where data are received 22. // * MQTT channel where data is send 23. // Used input pins 24. // pulse counting pin 19 = INT2 25. // AD input pin = A7 26. // AD input pin = A8 27. // AD input pin = A9 28. // AD input pin = A9 29. // AD input pin = A 30. // AD input pin = A9 31. // AD input pin = A 32. // AD input pin = A9 33. // AD input pin = A 34. // AD input pin = A9’ 35. // volatile variables 36. // SETUP section 37. // put your setup code here, to run once: 38. // Serial monitor baudrate = 9600 39. // Counting Pin 2 as input 40. // Pull up resistors ON 41. // interrupt routine defination 42. // Display size defination 20 char 4 row 43. // set curosor to left upper corner 44. // print to lCD 45. // print to serial monitor 46. // initialize Ethernet connection 47. // get IP number from DHCP 48. // connect to MQTT server 49. LOOP section 50. // put your main code here, to run repeatedly: 51. // contious loop 52. // keep MQTT connection online 53. // Voltage measuring 54. // clear 2 rd row 55. // Voltage measuring 56. // clear 3 rd row 57. // Voltage measuring 58. // clear 4 rd row 59. // 1.23V = dark 60. // 4.5 V bright 4.5-1.23=3.27 61. // print scaled value to LCD 62. // print voltage to serial monitor 63. // SEND to MQTT server measured voltage 64. // 2.5V = 0 C 65. // 4.5V = 20 C 66. // print scaled value to LCD 67. // print voltage to serial monitor 68. // SEND to MQTT server measured voltage 69. // print scaled value to LCD 70. // print voltage to serial monitor 71. // SEND to MQTT server measured voltage 72. // print scaled value to LCD 73. // print voltage to serial monitor 74. // read AD converter value, put readed value to variable measure 75. // scale measure value to 0.0-5.0 76. LOOP END 77. //Read pulses with interrupt service 78. // time to count pulses (in milliseconds) 79. // start time from millisocond counter 80. // wait pulse counting time 1 sec 81. // Re connect if connection is lost 82. // keep MQTT connection online

83. //input pin interrupt routine 84. //Interrupt service 85. // increment pulse counter each time when input pin moves from "0" to "1" 86. // Ethernet routines 87. // GET IP number from DHCP server 88. //MQTT Routines 89. // Send message 90. // check sign 91. // create message with header and data 92. // Print message to serial monitor 93. // send message to MQTT server 94. // Re connect if connection is lost 95. // Re Connection 96. // wait for reconnecting 97. // Receive incoming MQTT message 98. // copy the payload content into a char* 99. // copy received message to receiv_string 100. // print reveived message to LCD 101. // MQTT server connection 102. // Print MQTT server IP number to Serial monitor 103. // Print MQTT server IP number to LCD 104. // check if allready connected 105. // connection to MQTT server 106. // Connection is OK 107. // subscript to in topic 108. // error in connection

- 5   RAJAPINNAT

- SW

- Arduino

- Raspberry

- MQTT

- MySQL

- REST-API

- Web UI

- HTML

- PHP

- JS

- C++

- Ethernet moduuli

- HW Elektroniikka

- Anturi mittaamiseen (BMP180)

- Alusta jossa :

- Regulaattori (+8V, 5V, 3.3V)

- Arduino nano – ohjelma valmiina

- Ethernet moduuli

- Koekytkentäalusta kytkennän tekoon

- LCD-näyttö

- Näppäimistö

- 6   KAAVIOT, PIIRRUSTUKSET JA KUVAT

- 6   LIITTEET

- Ohjelmakoodit kommentteineen

- /*

- *    Arduino W5100 EtherCard ICT 2016 alkukoodit - edit 15.1.2016 KN

- */

- #define DEBUG

- #define SERIAL_ON

- #include <SPI.h>

- #include <Ethernet.h>

- #include <EthernetUdp.h>

- #include <PubSubClient.h>

- #include <TimerOne.h>

- #include <avr/interrupt.h>

- #include <avr/io.h>

- #include <LiquidCrystal.h>

- #include <Time.h>

- #include <TimeLib.h>

- #include <avr/pgmspace.h>

- //#include <raspIP.h>

- #include <Adafruit_BMP085.h>

- #include <Wire.h>

- //

- //

- //   Kanavanumerot

- //

- //                    A     B

- //

- // Tuulen nopeus 1    1    21

- // Tuulen suunta 1    2    22

- // Lampotila 1        3    23

- // Sademaara 1        4    24

- // Kosteus sisa       5    25

- // Kostesus ulko      6    26

- // Tuulen suunta2     7    27

- // Tuulen nopeus2     8    28

- // Lampotila2         9    29

- // Sademaara2         10   30

- // Ilmanpaine1        11   31

- // Ilmanpaine2        12   32

- /////////////////////////////////////////////////////////////////////////////////////////////

- /// signaali                                                                              ///

- /// ILmanpaine 1                                                                          ///

- /////////////////                                                                         ///

- ///  Kanava nro                                                                           ///

- ///

- #define   kanava         32       // A                                                    ///

- #define   kanava_numero "32"                                                              ///

- ///

- ////////////////                                                                          ///

- ///MAC nro       // eka kanava MAC6 A luokka 0x21-0x32                                    ///

- //                                  B luokka 0x41-0x52                                    ///

- #define  mac_6    0x52                                                                    ///

- ///

- /////////////////////////////////////////////////////////////////////////////////////////////

- Adafruit_BMP085 bmp;

- #define mittaus_m 1

- LiquidCrystal lcd(8,7,6,5,4,3);

- static uint8_t mymac[6] = { 0x54,0x55,0x58,0x10,0x00,mac_6 };

- unsigned int localPort = 8888;       // local port to listen for UDP packets

- const int NTP_PACKET_SIZE = 48; // NTP time stamp is in the first 48 bytes of the message

- byte packetBuffer[ NTP_PACKET_SIZE]; //buffer to hold incoming and outgoing packets

- // A UDP instance to let us send and receive packets over UDP

- EthernetUDP Udp;

- #define inTopic    "ict1_in_2018"         // * MQTT channel where lelylan updates are received

- #define outTopic   "ict1_out_2018"        // * MQTT channel where lelylan updates are received

- char* deviceId     = "2016";             // * set your device id (will be the MQTT client username)

- char* deviceSecret = "tamk";         // * set your device secret (will be the MQTT client password)

- char* clientId     = "1234567890";             // * set a random string (max 23 chars, will be the MQTT client id)

- char website[] = "ntp.tamk.fi";

- //////////////////

- //// Raspberryn IP     Luetaan tiedostoa raspIP.h

- //#define  IP_1    192

- //#define  IP_2    168

- //#define  IP_3    1

- //#define  IP_4    2

- //// Raspberryn IP TAMK ICT

- //#ifdef IP_tamk

- #define  IP_1    10

- #define  IP_2    10

- #define  IP_3    206

- #define  IP_4    141

- //#endif

- /* Server settings */

- byte server[] = { IP_1, IP_2, IP_3,IP_4 }; // MQTT serveri osoite

- /* Ethernet configuration */

- EthernetClient ethClient;

- /* MQTT communication */

- void callback(char* topic, byte* payload, unsigned int length); // subscription callback

- PubSubClient client(server, 1883, callback, ethClient);         // mqtt client

- // Find list of servers at http://support.ntp.org/bin/view/Servers/StratumTwoTimeServers

- // Please observe server restrictions with regard to access to these servers.

- // luetaan tiedostosra raspIP.h

- //char website[]PROGMEM = "ntp2d.mcc.ac.uk";

- //char website[]PROGMEM = "ntp2c.mcc.ac.uk";

- //char website[]PROGMEM = "ntp.exnet.com";

- //char website[]PROGMEM = "ntp.cis.strath.ac.uk";

- //char website[]PROGMEM = "clock01.mnuk01.burstnet.eu";

- byte aika_nro=0;  // onnistuneiden ajan lukujen maara

- uint32_t paine;

- uint32_t timeLong;

- // Number of seconds between 1-Jan-1900 and 1-Jan-1970, unix time starts 1970

- // and ntp time starts 1900.

- void setup()

- {

- lcd.begin(16,2); // Näytön koon määrittely 16 merkkiä 2 riviä

- lcd.setCursor(0,0);

- lcd.print("14.3.2018 ");

- lcd.setCursor(0,1);

- //   1234567890123456

- lcd.print("Odotan IP nroa");

- #ifdef SERIAL_ON

- Serial.begin(9600);

- Serial.println( F("EtherCard/Nanode NTP Client" ) );

- #endif

- hae_IP();    // Kytkeydytään TAMK:n verkkoon ja pyydetään IP-osoite.

- if (!bmp.begin()) {

- Serial.println("Could not find a valid BMP085 sensor, check wiring!");

- }

- lcd.clear();

- } // Setup END

- ////////////////////////////////////////////////////

- /////////////////  LOOPIN ALKU

- ///////////////////////////////////////////////////

- void loop()

- {

- paine = bmp.readPressure();  // Adafruit kirjaston funktio joka palauttaa painedatan pascaleina.

- lcd.setCursor(0,0);

- lcd.print(float(paine) / 100);

- lcd.setCursor(8,0);

- lcd.print("mbar");

- Serial.print("Pressure = ");

- Serial.print(paine);

- Serial.println(" Pa");

- lcd.setCursor(0,1);

- printTime();            // Noutaa palvelimelta kellonajan ja tulostaa sen LCD-näytölle muodossa hh:mm:ss

- Connect_MQTT_server();  // Otetaan yhteys MQTT serveriin

- send_message(paine);     // Ottaa parametrina painelukeman ja lähettää sen MQTT serverille

- Serial.println();

- delay(500);

- }  // Loop End

- void printTime()

- {

- laheta_pyynto();  // Lähettää ajanhakupyynnön palvelimelle.

- time_t aika = timeLong;           //Parsii ajanhakupyynnöstä saadun datan tulostettavaan aikamuotoon.

- int tunnit = hour(aika) + 2;

- int minuutit = minute(aika);

- int sekunnit = second(aika);

- if (tunnit < 10)

- {

- lcd.print("0");

- lcd.print(tunnit);

- }

- else

- {

- lcd.print(tunnit);

- }

- lcd.print(":");

- if (minuutit < 10)

- {

- lcd.print("0");

- lcd.print(minuutit);

- }

- else

- {

- lcd.print(minuutit);

- }

- lcd.print(":");

- if (sekunnit < 10)

- {

- lcd.print("0");

- lcd.print(sekunnit);

- }

- else

- {

- lcd.print(sekunnit);

- }

- }

- ///////////////////////////////////////////

- //////// IP numeron haku

- //////////////////////////////////////////

- void hae_IP(void)

- {

- byte rev=1;

- rev=Ethernet.begin( mymac);

- #ifdef SERIAL_ON

- Serial.print( F("\nW5100 Revision ") );

- #endif

- if ( rev == 0){

- #ifdef SERIAL_ON

- Serial.println( F( "Failed to access Ethernet controller" ) );

- #endif

- // 1234567890123456

- lcd.setCursor(0,1); lcd.print("FAIL E cont conn");

- }

- #ifdef SERIAL_ON

- Serial.println( F( "Setting up DHCP" ));

- Serial.print("Connected with IP: ");

- Serial.println(Ethernet.localIP());

- #endif

- lcd.setCursor(0,0);

- lcd.print("IP=");

- lcd.print(Ethernet.localIP());

- }

- ///////////////////////////////////////////////

- //////////  ajan hakupyynto

- ///////////////////////////////////////////////

- void laheta_pyynto(void)

- {

- byte DSNError=1;

- Udp.begin(localPort);

- sendNTPpacket(website); // send an NTP packet to a time server

- delay(200);

- if (Udp.parsePacket())

- {

- // We've received a packet, read the data from it

- Udp.read(packetBuffer, NTP_PACKET_SIZE); // read the packet into the buffer

- // the timestamp starts at byte 40 of the received packet and is four bytes,

- // or two words, long. First, extract the two words:

- unsigned long highWord = word(packetBuffer[40], packetBuffer[41]);

- unsigned long lowWord = word(packetBuffer[42], packetBuffer[43]);

- // combine the four bytes (two words) into a long integer

- // this is NTP time (seconds since Jan 1 1900):

- unsigned long secsSince1900 = highWord << 16 | lowWord;

- // now convert NTP time into everyday time:

- // Unix time starts on Jan 1 1970. In seconds, that's 2208988800:

- const unsigned long seventyYears = 2208988800UL;

- // subtract seventy years:

- unsigned long epoch = secsSince1900 - seventyYears;

- timeLong=epoch;

- }

- else

- {

- delay(10000);

- Ethernet.maintain();

- }

- aika_nro++;     // ajan luku onnistui

- }    /// ajan paivitys loppuuu

- // send an NTP request to the time server at the given address

- void sendNTPpacket(char* address) {

- // set all bytes in the buffer to 0

- memset(packetBuffer, 0, NTP_PACKET_SIZE);

- // Initialize values needed to form NTP request

- // (see URL above for details on the packets)

- packetBuffer[0] = 0b11100011;   // LI, Version, Mode

- packetBuffer[1] = 0;     // Stratum, or type of clock

- packetBuffer[2] = 6;     // Polling Interval

- packetBuffer[3] = 0xEC;  // Peer Clock Precision

- // 8 bytes of zero for Root Delay & Root Dispersion

- packetBuffer[12]  = 49;

- packetBuffer[13]  = 0x4E;

- packetBuffer[14]  = 49;

- packetBuffer[15]  = 52;

- // all NTP fields have been given values, now

- // you can send a packet requesting a timestamp:

- Udp.beginPacket(address, 123); //NTP requests are to port 123

- Udp.write(packetBuffer, NTP_PACKET_SIZE);

- Udp.endPacket();

- }

- /////////////////////////////////////////////////////////////////////////////////

- //////////////// MQTT RUTIINIT

- ////////////////////////////////////////////////////////////////////////////////

- /// Laheta sanoma

- void send_message(uint32_t data)

- {

- char bufa[25];

- sprintf(bufa,"32:+%06lu",(unsigned long)data);

- #ifdef SERIAL_ON

- Serial.println( bufa );

- #endif

- client.publish(outTopic,bufa);

- }

- /* Receive MQTT message and confirm the physical change */

- void callback(char* topic, byte* payload, unsigned int length)

- {

- // copu the payload content into a char*

- char* json;

- json = (char*) malloc(length + 1);

- memcpy(json, payload, length);

- json[length] = '\0';

- lcd.setCursor(0,0);

- lcd.print(json);

- #ifdef SERIAL_ON

- Serial.println( json );

- #endif

- free(json);

- }

- /* MQTT server connection */

- void Connect_MQTT_server()

- {

- // add reconnection logics

- if (!client.connected())

- {

- // connection to MQTT server

- if (client.connect(clientId, deviceId, deviceSecret))

- {

- lcd.setCursor(0,1);

- //0123456789012345//

- lcd.print("Connect         ");

- #ifdef SERIAL_ON

- Serial.println(" Connected" );

- #endif

- MQTTSubscribe(); // topic subscription

- }

- }

- }

- /* MQTT subscribe */

- void MQTTSubscribe()

- {

- client.subscribe(inTopic);

- }